# Quiz — 1.1.1 Structure and Function of the Processor - ANSWERS

## Answer key

Section A (8 marks — 1 each)

C — The Program Counter always holds the address of the next instruction; it is incremented each cycle.

B — The Memory Data Register holds data/instruction just fetched or about to be written. (Remember: MAR = address, MDR = data.)

B — The address bus is unidirectional, CPU → memory only.

C — After PC → MAR, a read signal is placed on the control bus and the instruction returns via the data bus into the MDR.

C — The CU decodes instructions and generates control signals to synchronise and direct the CPU.

B — Cache is small, very fast memory on/near the CPU (not part of RAM) holding frequently/recently used data.

B — Von Neumann uses one shared memory and bus for both data and instructions.

B — Pipelining overlaps fetch/decode/execute of consecutive instructions to raise throughput.

Section B (12 marks)

[3] — 1 mark each (full name + role):

MAR = Memory Address Register — holds the address of the location to be read from / written to.

MDR = Memory Data Register — holds the data/instruction just fetched, or about to be written, to memory.

CIR = Current Instruction Register — holds the current instruction while it is decoded/executed.

[2] — 1 mark each: Performs arithmetic operations (+ − × ÷); performs logic operations / comparisons (AND, OR, NOT, compare) and shifts; (result is placed in the ACC). Any two distinct points.

[2] — 1 mark: higher clock speed = more pulses/cycles per second, so more fetch–decode–execute cycles completed per second / more instructions processed per second. 1 mark: limited by heat / power consumption (overheating).

[3] — 1 mark per valid difference (max 3). Examples:

Von Neumann = one shared memory; Harvard = separate memories for data and instructions.

Von Neumann = shared bus; Harvard = separate buses.

Von Neumann can suffer a bottleneck (cannot fetch instruction and access data at once); Harvard can do both simultaneously.

Von Neumann is simpler/cheaper/more flexible; Harvard is more complex/costly.

Von Neumann used in general-purpose PCs; Harvard used in embedded/microcontrollers/DSPs.

[2] — 1 mark: it uses a single unified main memory (Von Neumann style) for programs and data; 1 mark: but has separate L1 instruction and data caches (Harvard style) close to the core, giving some simultaneous-access benefit.

Section C (5 marks)

[5] — Level-based / point marking, award up to 5 for a balanced, applied discussion. Indicative content:

Higher clock speed (X) means each individual core completes more FDE cycles per second — good for the sequential / single-threaded parts of a game (e.g. game logic, physics dependencies).

More cores (Y) allow parallel processing — multiple tasks (rendering prep, AI, audio, physics) run simultaneously — but only if the game engine is written to use multiple threads.

Larger cache (Y) reduces slow RAM fetches (fewer cache misses), improving throughput for frequently accessed data/instructions.

Trade-off: if the game is poorly multi-threaded, X's higher clock speed may win; modern game engines are increasingly parallel, favouring Y.

Justified conclusion linking the choice to the workload (e.g. "Y is likely better for a modern, multi-threaded engine because…").

Max 4 if no justified judgement/conclusion is made.

Total: 8 + 12 + 5 = 25