# Quiz — 1.4.3 Boolean Algebra

Name: ____   Date: __   Mark: ____ / 30

OCR H446 · Component 01 · 1.4.3 — show all working

## Section A — Multiple choice (8 marks)

1. For which input combination does a 2-input AND gate output 1?

A. A=0, B=0

B. A=1, B=0

C. A=0, B=1

D. A=1, B=1

Your answer: ☐

2. Which logic gate outputs 1 only when its two inputs differ?

A. AND

B. OR

C. XOR

D. NOR

Your answer: ☐

3. Which two gates are described as universal (any circuit can be built from them alone)?

A. AND and OR

B. NAND and NOR

C. XOR and NOT

D. OR and NOT

Your answer: ☐

4. De Morgan's law states that ¬(A + B) is equivalent to:

A. ¬A + ¬B

B. ¬A · ¬B

C. A · B

D. ¬(A · B)

Your answer: ☐

5. Which Boolean identity is ¬(¬A) = A?

A. Idempotent

B. Absorption

C. Double negation

D. Complement

Your answer: ☐

6. In a half adder, the Sum output is given by:

A. A · B

B. A + B

C. A ⊕ B

D. ¬(A · B)

Your answer: ☐

7. What is the role of a D-type flip-flop?

A. To add two bits together

B. To store one bit of data on a clock edge

C. To invert its input

D. To convert binary to hexadecimal

Your answer: ☐

8. When grouping 1s on a Karnaugh map, groups must be:

A. Always exactly size 3

B. As small as possible

C. Powers of two (1, 2, 4, 8…) and as large as possible

D. Only ever a single cell

Your answer: ☐

## Section B — Calculations & short answer (17 marks)

9. Complete the truth table for the expression Q = ¬(A · B) + (A · C). [5]

| A | B | C | A·B | ¬(A·B) | A·C | Q |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 |  |  |  |  |
| 0 | 0 | 1 |  |  |  |  |
| 0 | 1 | 0 |  |  |  |  |
| 0 | 1 | 1 |  |  |  |  |
| 1 | 0 | 0 |  |  |  |  |
| 1 | 0 | 1 |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |
| 1 | 1 | 1 |  |  |  |  |

10. Use De Morgan's law (and double negation) to simplify `¬(¬A + B)` to an expression with no outer bar. Show each step. [3]

Working space:

11. A half adder receives inputs A = 1 and B = 1. State the Sum and Carry outputs and the Boolean expression for each. [3]

Working space:

12. Complete this truth table for a half adder for all input combinations. [3]

| A | B | Sum (A⊕B) | Carry (A·B) |
| --- | --- | --- | --- |
| 0 | 0 |  |  |
| 0 | 1 |  |  |
| 1 | 0 |  |  |
| 1 | 1 |  |  |

13. Simplify the expression `A · (A + B)` using a Boolean identity, and name the law used. [3]

Working space:

## Section C — Exam-style question (5 marks)

14. A full adder adds three input bits: A, B and a carry-in (Cin), producing a Sum and a carry-out (Cout).

(a) Give the Boolean expression for the Sum output of a full adder. [1]

(b) Give the Boolean expression for the Cout output of a full adder. [2]

(c) Explain how a full adder differs from a half adder, and why full adders are needed to add multi-bit binary numbers. [2]

Working space: