# Quiz — 1.1.1 Structure and Function of the Processor

Name: ____   Date: __   Mark: ____ / 25

OCR H446 · Component 01 · 1.1.1

## Section A — Multiple choice (8 marks)

1. Which register always holds the address of the next instruction to be fetched?

A. MAR

B. MDR

C. PC

D. CIR

Your answer: ☐

2. Which register holds the data or instruction that has just been fetched from, or is about to be written to, memory?

A. MAR

B. MDR

C. ACC

D. CIR

Your answer: ☐

3. The address bus is best described as:

A. Bidirectional, carrying data and instructions

B. Unidirectional, carrying memory addresses from the CPU to memory

C. Bidirectional, carrying control signals

D. Unidirectional, carrying data from memory to the CPU

Your answer: ☐

4. During the fetch stage, immediately after the contents of the PC are copied to the MAR, what happens next?

A. The instruction is decoded into opcode and operand

B. The PC is reset to zero

C. A read signal is sent on the control bus and the instruction returns via the data bus to the MDR

D. The ALU performs the calculation and stores the result in the ACC

Your answer: ☐

5. What is the primary purpose of the Control Unit (CU)?

A. To perform arithmetic and logical operations

B. To store the immediate result of calculations

C. To decode instructions and generate control signals that coordinate the CPU

D. To provide fast storage of frequently used data

Your answer: ☐

6. Cache memory improves performance because it is:

A. Part of main RAM that has been reserved

B. Small, very fast memory on or near the CPU holding frequently/recently used data

C. A region of secondary storage used when RAM is full

D. A register inside the ALU

Your answer: ☐

7. Which statement about adding more cores is correct?

A. More cores always doubles performance

B. More cores only improve performance if the task can be parallelised and the software supports it

C. More cores reduce clock speed proportionally

D. More cores remove the need for cache

Your answer: ☐

8. Pipelining increases performance by:

A. Increasing the clock speed automatically

B. Overlapping the fetch, decode and execute stages of consecutive instructions

C. Doubling the width of the data bus

D. Storing more instructions in the accumulator

Your answer: ☐

## Section B — Short answer (12 marks)

9. State the full name of each register and give its role. [3]

MAR: ____

MDR: ____

CIR: ____

10. Describe the role of the ALU within the processor. [2]

11. A CPU has a 16-bit address bus. Calculate the maximum number of memory locations that can be addressed. Show your working. [2]

12. Explain how increasing the clock speed can improve CPU performance, and state one factor that limits how high clock speed can go. [2]

13. Explain the difference between the L1 and L3 cache in terms of speed and size, and state the order in which the CPU checks them. [2]

14. A jump (branch) instruction is executed. Explain what happens to the PC and why this can cause a problem for a pipelined processor. [1]

## Section C — Exam-style question (5 marks)

15. A games developer is comparing two processors. Processor X has a higher clock speed but a single core; Processor Y has a lower clock speed but four cores and a larger cache.

Discuss the factors the developer should consider when deciding which processor will give better performance for their game. (AO2/AO3) [5]