# Revision — 1.1.3 Input, Output and Storage Devices

OCR H446 — one-page revision summary

## Must know

Input devices send data into a system (mouse, keyboard, capacitive touchscreen, barcode scanner, microphone, sensor); output devices act on the world (monitor, printer, speakers, actuator).

There is no single "right" device — justify a choice against the scenario: environment, user/accessibility, accuracy, speed, cost, durability.

Printers: laser = fast, high-volume, low cost per page, sharp text (offices); inkjet = cheaper to buy, good occasional colour/photos, low volume (home); 3D = builds objects layer by layer.

Secondary storage is non-volatile — it retains data without power.

Magnetic (HDD, tape): magnetised patterns on a spinning platter/tape read by a R/W head — huge capacity, low £/GB, but slow and fragile (moving parts). Tape = serial access, ideal for cheap archive/backup.

Flash/SSD (SSD, USB, SD): EEPROM traps electrons in cells, no moving parts — fast, silent, low power, durable, portable; costlier/GB and finite write cycles (uses wear levelling).

Optical (CD/DVD/Blu-ray): a laser detects pits and lands via reflected light — very cheap and portable, but low capacity, slow and easily scratched.

RAM = volatile, read/write, holds running programs/data; ROM = non-volatile, normally read-only, holds bootstrap/BIOS firmware.

Virtual memory = when RAM is full, the OS moves less-used pages to a swap/paging file on secondary storage; excessive paging causes disk thrashing. Virtual (cloud) storage = remote servers accessed over a network (access anywhere, scalable, off-site backup; needs internet).

## Key terms

| Term | Definition |
| --- | --- |
| Volatile | Loses its contents when power is removed (e.g. RAM) |
| Non-volatile | Retains contents without power (secondary storage, ROM) |
| RAM | Volatile read/write memory holding currently running programs and data |
| ROM | Non-volatile, normally read-only memory holding bootstrap/BIOS firmware |
| Flash / SSD | Solid-state storage using EEPROM cells with no moving parts |
| Serial access | Data read/written in sequence (e.g. magnetic tape) |
| Virtual memory | Use of secondary storage as extra RAM when physical RAM is full |
| Disk thrashing | Excessive paging where the system spends most time swapping pages, not working |

## Worked example / how to — recommending a storage device

Scenario: portable storage for large video files, moved between sites, frequently rewritten, needing fast transfer and durability.

List the requirements — portable, large capacity, fast transfer, frequently rewritten, durable.

Recommend a device — an external flash/SSD (or USB flash drive).

Justify against each requirement — small/light and portable; fast transfer suits large video files; easily rewritable; durable (no moving parts to damage when carried).

Compare/reject alternatives — an HDD is cheaper per GB but has fragile moving parts; optical media is slow and low capacity — so flash best fits the stated needs.

## Exam tips

Distinguish input vs output carefully — an actuator is an output, a sensor/microphone is an input.

For "choose a device/storage" questions, link each characteristic to the scenario and compare ("whereas…"); listing generic pros/cons without applying them caps marks at AO1.

Keep the three "virtuals" separate: virtual memory (secondary storage as extra RAM) vs virtual/cloud storage (remote servers) — read which one the question means.

Be precise on mechanisms: magnetic = magnetised patterns, optical = pits and lands read by laser, flash = electrons trapped in cells, no moving parts.