# Week 03 (14 Sep) 1.1.2 Types of processor - Homework

Homework 1.1.2; revise for KA1 with the 1.1 Knowledge Organiser.

# Homework — 1.1.2 Types of Processor

Name: ______________________   Date: __________   Mark: ______ / 20

OCR H446 — set after the 1.1.2 lesson. Show your working.

## Part A — This week's topic (~14 marks)

1. Complete the comparison table of RISC and CISC processors by filling in the missing cells. [3]

| Feature | RISC | CISC |
| --- | --- | --- |
| Size of instruction set | Small set of simple instructions | (a) |
| Instruction length | (b) | Variable length |
| Clock cycles per instruction | (c) | Often several cycles |
| Where the complexity lies | (d) | In the hardware/circuitry |
| Program length / RAM needed | (e) | Shorter programs, less RAM needed |
| Typical power consumption | Lower | (f) |

2. Match each term to its definition by writing the correct letter in the Match column. Two definitions are not needed. [3]

| Term | Match |
| --- | --- |
| Multicore processor |  |
| Parallel processing |  |
| GPU |  |
| Co-processor |  |
| Pipelining |  |
| RISC |  |

| Letter | Definition |
| --- | --- |
| A | An extra processor that relieves the CPU of specific tasks, e.g. floating-point arithmetic or graphics |
| B | Carrying out several instructions/tasks at the same time |
| C | A processor with a very large number of simpler cores, designed to apply the same operation to many items of data at once |
| D | A CPU containing two or more processing units, each able to fetch, decode and execute its own instructions |
| E | A processor design with a small set of simple, fixed-length instructions that typically execute in one clock cycle |
| F | Overlapping the fetch, decode and execute stages of consecutive instructions |
| G | An architecture in which one shared memory holds both instructions and data |
| H | Small, very fast memory located on or near the CPU |

3. Each statement in the table below contains one factual error about GPUs. Identify the error in each and rewrite the statement correctly on the lines below. [3]

| # | Statement (each contains one error) |
| --- | --- |
| 1 | A GPU contains a small number of highly complex cores — far fewer than a typical CPU. |
| 2 | A GPU processes graphics efficiently because each core carries out a different instruction on the same single item of data. |
| 3 | GPUs cannot be used for any task other than producing graphics. |

4. A smartphone manufacturer uses a RISC (ARM) processor rather than a CISC (x86) processor. Give two reasons, linked to a smartphone, why RISC is the better choice. [2]

5. Tick True or False for each statement. If a statement is false, write a correction in the final column. [3] (AO1/AO2)

| Statement | True / False | Correction if false |
| --- | --- | --- |
| Any program automatically runs faster on a processor with more cores. |  |  |
| A co-processor system uses an additional processor to take over specific tasks, such as floating-point calculations, from the CPU. |  |  |
| In parallel processing, the computer still completes every instruction one after another, in strict sequence. |  |  |

## Part B — Retrieval: keep it fresh (~6 marks)

6. Explain why the Von Neumann architecture can suffer from a performance bottleneck, and state how the Harvard architecture avoids it. [3] (1.1.1)

7. State two factors (other than number of cores) that affect the performance of a CPU, and for each give its effect. [2] (1.1.1)

8. Name the register that holds the current instruction while it is being decoded and executed. [1] (1.1.1)