# Quiz — 1.1.2 Types of Processor - ANSWERS

## Answer key

Section A (8 marks — 1 each)

B — RISC = few, simple, fixed-length instructions, aiming for one cycle each.

B — In CISC the complexity is in the hardware (microcode); in RISC it is in software/compiler.

B — RISC/ARM is used in mobile/embedded devices for low power consumption.

B — GPUs apply the same operation to large data blocks in parallel (data parallelism).

B — Training a neural network is a non-graphics (GPGPU) use; rendering pixels is standard graphics.

B — Data dependency (a step needs the previous step's result) prevents effective parallelisation.

B — More cores help only if the task is parallelisable and software supports it.

A — Simple, fixed-length, single-cycle instructions keep the pipeline stages uniform, so instructions flow through without stalling.

Section B (12 marks)

[2] — 1 mark per difference (max 2). Examples: CISC has many complex variable-length instructions vs RISC few simple fixed-length; complexity in hardware (CISC) vs software/compiler (RISC); RISC easier to pipeline; RISC lower power; CISC used in x86 desktops vs RISC in ARM mobile/embedded.

[2] — 1 mark: a GPU has thousands of small cores; 1 mark: it can process many pixels/vertices in parallel (the same operation on large data sets simultaneously), so rendering is much faster than doing it sequentially.

[1] — A processor (chip) containing two or more independent cores, each able to run its own fetch–decode–execute cycle, allowing simultaneous processing.

[2] — 1 mark: there is a data dependency — the search needs the completed (sorted) list, so it cannot begin until the sort finishes. 1 mark: the two stages must run sequentially, so extra cores cannot speed up the overall sequence (only the sort itself might be parallelised); benefit is limited by the serial fraction.

[3] — 1 mark each (max 3), any non-graphics (GPGPU) use: machine learning / training neural networks; scientific/numerical simulations and modelling; image/video processing; data mining/analysis; cryptocurrency mining / brute-force hashing.

[2] — 1 mark: RISC's simple, fixed-length instructions need less complex circuitry, so the processor uses less power / generates less heat. 1 mark: applied link — lower power extends the tracker's battery life / allows a small fanless device to run cool.

Section C (5 marks)

[5] — Point/level marking, up to 5 for a justified, applied discussion. Indicative content:

The workload applies the same calculation to millions of data points — this is highly data-parallel, with little branching or dependency between points.

A GPU has thousands of small cores and excels at performing the same operation on large data sets in parallel (data parallelism / GPGPU), so it can process far more data points simultaneously.

A multicore CPU has fewer, more powerful cores better suited to sequential, branch-heavy, dependency-laden tasks — fewer parallel lanes for this kind of work.

Caveat: the software must be written to use the GPU; very branchy or strongly dependent calculations would suit the CPU better.

Justified conclusion: for this uniform, massively data-parallel simulation the GPU is the better choice because it maximises throughput across many data points.

Max 4 if no clear justified judgement is given.

Total: 8 + 12 + 5 = 25