# Quiz — 1.3.3 Networks

Name: ____   Date: __   Mark: ____ / 25

OCR H446 · Component 01 · 1.3.3

## Section A — Multiple choice (8 marks)

1. Which best describes a LAN?

A. A network spanning cities or countries using third-party infrastructure

B. A network over a small geographical area where the organisation owns the hardware

C. A network with no physical hardware

D. The largest WAN in the world

Your answer: ☐

2. In a peer-to-peer network:

A. A central server provides all services

B. All devices are equal and can act as client and server

C. There is always a single point of failure

D. Security is centrally managed

Your answer: ☐

3. A disadvantage of a star topology is that:

A. A single cable fault brings down every device

B. The central switch/hub is a single point of failure

C. It cannot use a switch

D. It requires every node to connect to every other node

Your answer: ☐

4. In packet switching, packets are:

A. Sent along one dedicated reserved path

B. Routed independently and reassembled at the destination

C. Never reassembled

D. Only used for telephone calls

Your answer: ☐

5. Which protocol is used to send email?

A. POP3

B. IMAP

C. SMTP

D. FTP

Your answer: ☐

6. The DNS is responsible for:

A. Encrypting web traffic

B. Translating domain names into IP addresses

C. Splitting data into packets

D. Assigning MAC addresses

Your answer: ☐

7. Which device connects two different networks and routes using IP addresses?

A. Switch

B. Router

C. NIC

D. Wireless access point

Your answer: ☐

8. At which TCP/IP layer are source and destination IP addresses added?

A. Application

B. Transport

C. Network/Internet

D. Link/Data link

Your answer: ☐

## Section B — Short answer (12 marks)

9. State two advantages of a client-server network over a peer-to-peer network. [2]

10. TCP/IP layer matching. Match each item (A–D) to the correct layer by writing the layer name next to it. [4]

A. Adds the MAC address / handles the physical connection → ___

B. Adds source and destination IP addresses and routes packets → ___

C. Splits data into packets, manages ports and end-to-end delivery → ___

D. Provides services for user applications (e.g. HTTP, SMTP) → ___

11. Explain the difference between a switch and a router. [2]

12. Describe what a proxy server does and give one benefit of using one. [2]

13. Explain one reason packet switching is more resilient than circuit switching. [2]

## Section C — Exam-style question (5 marks)

14. A user types a web address into their browser and requests a secure web page over the internet.

Describe what happens, referring to: the role of DNS, how the data is broken up and routed across the network, and one piece of hardware involved. [5]

## Answer key

Section A (8 marks, 1 each)

B — LAN = small area, single site, organisation-owned hardware.

B — In P2P all peers are equal, acting as both client and server.

B — The central switch/hub is the single point of failure in a star.

B — Packets are routed independently and reassembled at the destination.

C — SMTP sends mail; POP3/IMAP retrieve it.

B — DNS translates domain names to IP addresses.

B — A router connects different networks and routes using IP.

C — The Network/Internet layer adds IP addressing and routes packets.

Section B (12 marks)

[2] Any two for 1 mark each: centralised security/backups; easier to manage/update centrally; scales well to many users; central data storage/control.

[4] (1 mark each)

A → Link / Data link layer

B → Network / Internet layer

C → Transport layer

D → Application layer

[2] 1 mark: a switch connects devices within a single LAN and forwards frames using MAC addresses. 1 mark: a router connects different networks (e.g. LAN to internet) and forwards packets using IP addresses.

[2] 1 mark: a proxy acts as an intermediary between client and server, forwarding requests/responses. 1 mark for one benefit: hides the user's IP address, caches pages to speed up access, or filters/logs/monitors traffic.

[2] 1 mark: packets can be routed independently along different routes. 1 mark: if one route/node fails, packets can be re-routed around it (no single dedicated path to break), so the communication still gets through.

Section C (5 marks — award up to 5 from)

DNS translates the typed domain name into the server's IP address (querying DNS servers, results may be cached). [1]

The browser (client) sends a request to that IP address; HTTPS is used so the data is encrypted in transit. [1]

The data is split into packets at the transport layer, each given a header (e.g. sequence number, ports). [1]

Packets are given source/destination IP addresses and routed independently across the network, then reassembled in order at the destination. [1]

A router (and/or switch/NIC) forwards the packets between networks toward the destination using IP addresses. [1]

Total: 8 + 12 + 5 = 25 marks