# Revision — 1.4.3 Boolean Algebra

OCR H446 — one-page revision summary

## Must know

Gates: NOT ¬A (invert); AND A·B (both 1); OR A+B (either 1); XOR A⊕B (inputs differ); NAND ¬(A·B); NOR ¬(A+B). NAND and NOR are universal — any circuit can be built from either alone.

De Morgan's laws: ¬(A·B) = ¬A + ¬B and ¬(A+B) = ¬A · ¬B. "Break the bar, change the sign" — negate each variable and swap AND↔OR.

Core identities: Identity A·1=A, A+0=A; Null A·0=0, A+1=1; Idempotent A·A=A, A+A=A; Complement A·Ā=0, A+Ā=1; Double negation ¬(¬A)=A.

Distributive: A·(B+C) = A·B + A·C and A+(B·C) = (A+B)·(A+C). Absorption: A + A·B = A and A·(A+B) = A.

Simplify expressions by applying identities step by step; tidy with double negation. Use to reduce gate count.

Karnaugh (K-)maps: plot outputs, group adjacent 1s in blocks that are powers of two (1, 2, 4, 8…), as large as possible, may wrap/overlap → minimal sum-of-products.

Half adder (2 bits, no carry-in): Sum = A ⊕ B, Carry = A · B.

Full adder (A, B, Cin) = two half adders + OR: Sum = A ⊕ B ⊕ Cin, Cout = (A·B) + (Cin·(A⊕B)). Chain them → ripple-carry adder for multi-bit addition.

D-type flip-flop: edge-triggered 1-bit memory (sequential logic). On the active clock edge it stores input D and presents it at Q until the next edge; synchronises data to the clock. Uses: registers, counters, memory cells.

Combinational logic = output depends only on current inputs; sequential logic = output depends on inputs plus stored state.

## Key terms

| Term | Definition |
| --- | --- |
| XOR | Gate outputting 1 only when its two inputs differ |
| Universal gate | Gate (NAND or NOR) from which any circuit can be built |
| De Morgan's law | ¬(A·B)=¬A+¬B, ¬(A+B)=¬A·¬B — negate each term and swap operator |
| Absorption law | A + A·B = A; A·(A+B) = A |
| Karnaugh map | Grid grouping adjacent 1s (powers of 2) to minimise an expression |
| Half adder | Adds 2 bits → Sum (A⊕B), Carry (A·B); no carry-in |
| Full adder | Adds 3 bits (A, B, Cin) → Sum + Cout; chainable |
| D-type flip-flop | Edge-triggered 1-bit memory storing D on a clock edge |

## Worked example / how to

Simplify `¬(¬A + B)` using De Morgan's law + double negation:

Apply De Morgan's to the outer bar (break bar, change + to ·, negate each term): ¬(¬A + B) = ¬(¬A) · ¬B.

Apply double negation ¬(¬A) = A: → A · ¬B.

Result: `A · ¬B`.

## Exam tips

De Morgan's: negate each variable and change the operator — not just one side; finish by tidying double negations.

State Sum = A⊕B and Carry = A·B exactly; the half adder has no carry-in, the full adder adds three bits.

For simplification, name the law used at each step (e.g. absorption, distributive) — it earns marks.

K-map groups must be powers of two and as large as possible; they may wrap around and overlap.