# Homework — 1.1.1 Structure and Function of the Processor - ANSWERS

## Mark scheme

Part A (14 marks)

1. [4] — 2 correct cells per mark. Completed table:

| Register | Full name | Role |
| --- | --- | --- |
| PC | Program Counter | Holds the address of the next instruction to be fetched (incremented each cycle) |
| ACC | Accumulator | Stores the immediate result of ALU calculations |
| MAR | Memory Address Register | Holds the address of the location in memory to be read from / written to |
| CIR | Current Instruction Register | Holds the instruction currently being decoded and executed |

2. [2] — Correct order:

| Event | Order |
| --- | --- |
| Instruction transferred from memory to the MDR along the data bus | 2 |
| CU decodes the instruction in the CIR into opcode and operand | 5 |
| Address held in the PC is copied into the MAR | 1 |
| Contents of the MDR are copied into the CIR | 4 |
| PC is incremented by 1 | 3 |

All five correctly placed = 2 marks; three or four correctly placed = 1 mark. (Accept the PC increment at position 2, 3 or 4 provided the other events keep their relative order.)

3. [3] — 1 mark per fully correct row:

| PC | CIR | ACC | Contents of address 6 |
| --- | --- | --- | --- |
| 1 | LOAD 4 | 7 | 0 |
| 2 | ADD 5 | 12 | 0 |
| 3 | STORE 6 | 12 | 12 |

(Working: LOAD 4 copies 7 into ACC; ADD 5 gives 7 + 5 = 12; STORE 6 writes 12 to address 6. The PC is incremented during each fetch, so it reads 1, 2, 3.)

4. [3] — 2 correct blanks per mark: (a) clock; (b) cache; (c) RAM; (d) pipelining; (e) executed; (f) core. (Distractors not used: interrupt, register, ROM.)

5.

(a) [1] — 2²⁰ = 1,048,576 locations. (Accept "1M" / "1 mega-location".)

(b) [1] — More bits are carried per transfer, so more data/instructions move per cycle → higher throughput / faster data movement.

Part B (6 marks)

6. [3] — 1 mark per row: Address bus = B; Data bus = A; Control bus = C. (D and E are distractors.)

7. [2] — 1 mark: cache is small, very fast memory on/near the CPU holding frequently/recently used data/instructions (not part of RAM). 1 mark: it reduces the need to fetch from slower RAM, so data is retrieved faster / fewer slow main-memory accesses.

8. [1] — Any one (1): Von Neumann uses a single shared memory (and bus) for both instructions and data, whereas Harvard uses separate memories (and buses) for instructions and data; Harvard can fetch an instruction and access data simultaneously, Von Neumann cannot.

Total: 14 + 6 = 20