# Quiz — 1.3.3 Networks - ANSWERS

## Answer key

Section A (8 marks, 1 each)

B — LAN = small area, single site, organisation-owned hardware.

B — In P2P all peers are equal, acting as both client and server.

B — The central switch/hub is the single point of failure in a star.

B — Packets are routed independently and reassembled at the destination.

C — SMTP sends mail; POP3/IMAP retrieve it.

B — DNS translates domain names to IP addresses.

B — A router connects different networks and routes using IP.

C — The Network/Internet layer adds IP addressing and routes packets.

Section B (12 marks)

[2] Any two for 1 mark each: centralised security/backups; easier to manage/update centrally; scales well to many users; central data storage/control.

[4] (1 mark each)

A → Link / Data link layer

B → Network / Internet layer

C → Transport layer

D → Application layer

[2] 1 mark: a switch connects devices within a single LAN and forwards frames using MAC addresses. 1 mark: a router connects different networks (e.g. LAN to internet) and forwards packets using IP addresses.

[2] 1 mark: a proxy acts as an intermediary between client and server, forwarding requests/responses. 1 mark for one benefit: hides the user's IP address, caches pages to speed up access, or filters/logs/monitors traffic.

[2] 1 mark: packets can be routed independently along different routes. 1 mark: if one route/node fails, packets can be re-routed around it (no single dedicated path to break), so the communication still gets through.

Section C (5 marks — award up to 5 from)

DNS translates the typed domain name into the server's IP address (querying DNS servers, results may be cached). [1]

The browser (client) sends a request to that IP address; HTTPS is used so the data is encrypted in transit. [1]

The data is split into packets at the transport layer, each given a header (e.g. sequence number, ports). [1]

Packets are given source/destination IP addresses and routed independently across the network, then reassembled in order at the destination. [1]

A router (and/or switch/NIC) forwards the packets between networks toward the destination using IP addresses. [1]

Total: 8 + 12 + 5 = 25 marks