# Mini Paper — 1.1 Processors, Input, Output & Storage

Time: ~35 minutes · Total: 30 marks · Answer all questions.

Mark your work against the mark scheme at the end; aim for ~1 minute per mark.

## Questions

Q1. State the function of each of the following CPU registers: (a) the Memory Address Register (MAR), and (b) the Memory Data Register (MDR). [2] (AO1)

Q2. Describe the register transfers that take place during the fetch phase of the Fetch–Decode–Execute cycle. [4] (AO1)

Q3. A CPU has three levels of cache (L1, L2 and L3).

(a) Explain how cache improves the performance of the CPU. [2] (AO1)

(b) Describe one difference between L1 cache and L3 cache. [2] (AO1)

Q4. A team is designing a microcontroller for a washing machine and is deciding between a Von Neumann and a Harvard architecture. Explain one advantage of choosing a Harvard architecture for this embedded device. [3] (AO2)

Q5. Compare CISC and RISC processors. In your answer refer to the instruction set and where complexity lies. [3] (AO2)

Q6. A research group trains large neural networks that apply the same calculation across huge data sets. Explain why a GPU is better suited to this work than a general-purpose CPU. [3] (AO2)

Q7. Explain what is meant by virtual memory and describe one drawback of relying on it heavily. [3] (AO1/AO2)

Q8. A media company needs secondary storage for two purposes:

(a) the internal drive of laptops used by journalists who travel and film on location, and

(b) long-term, low-cost archiving of very large volumes of old footage that is rarely accessed.

For each purpose, recommend a suitable storage technology (magnetic, flash or optical) and justify your choice, evaluating the trade-offs against the requirements. [8] (AO3)

## Mark scheme

Q1 — [2] (AO1)

(a) MAR holds the address of the memory location to be read from or written to (1).

(b) MDR holds the data or instruction just fetched from, or about to be written to, memory (1).

Examiner tip: Keep them separate — MAR = address, MDR = data; swapping them is the most common lost mark here.

Q2 — [4] (AO1)

The address held in the PC is copied to the MAR (1).

A read signal is sent on the control bus and the instruction at that address is fetched along the data bus into the MDR (1).

The PC is incremented so it points to the next instruction (1).

The contents of the MDR are copied into the CIR, ready for decoding (1).

Examiner tip: Don't forget the PC increment and the MDR→CIR copy — both are routinely dropped.

Q3 — [4] (AO1)

(a) Cache is small, very fast memory on/near the CPU storing frequently/recently used instructions and data (1); the CPU can fetch these without accessing slower main memory (RAM), reducing waiting time (1).

(b) Any one clear difference for 2 marks, e.g. L1 is smaller than L3 (1) and L1 is faster / on the core whereas L3 is slower / often shared between cores (1).

Examiner tip: Say cache is "on/near the CPU" — calling it "part of RAM" loses the mark.

Q4 — [3] (AO2)

Harvard architecture has separate memories and buses for instructions and data (1).

This means an instruction can be fetched at the same time as data is read/written (avoids the Von Neumann bottleneck) (1).

For the washing machine this gives faster, more predictable execution of its fixed control program, which suits a real-time embedded device (1).

Examiner tip: The mark for application is tying "simultaneous access" back to the embedded/real-time scenario, not just defining Harvard.

Q5 — [3] (AO2) (max 3)

CISC has a large set of complex, variable-length instructions, whereas RISC has a small set of simple, fixed-length instructions (1).

In CISC the complexity is in hardware; in RISC the complexity is in software/the compiler (more instructions per task) (1).

RISC's uniform single-cycle instructions are easier to pipeline and use less power, whereas CISC instructions may take several cycles (1).

Examiner tip: Actually compare using "whereas"/"compared to"; two separate descriptions caps your marks.

Q6 — [3] (AO2) (max 3)

A GPU has many (thousands of) cores designed for parallel processing (1).

Neural network training applies the same operation to large blocks of data (data parallelism), so the independent calculations run simultaneously across the cores (1).

A CPU has only a few cores optimised for sequential tasks, so it would be slower for this highly parallel workload (1).

Examiner tip: Name the data-parallel nature of the work — "GPUs are just faster" without explaining why scores poorly.

Q7 — [3] (AO1/AO2)

Virtual memory is when RAM is full and the OS uses a portion of secondary storage (a swap/paging file) as if it were extra RAM (1), moving pages not currently needed out of RAM and back when required (1).

Drawback: secondary storage is far slower than RAM, so heavy paging causes disk thrashing and slows the system (1).

Examiner tip: Don't confuse virtual memory (disk used as overflow RAM) with cloud/virtual storage (remote servers).

Q8 — [8] (AO3) (levelled, max 8)

Indicative content — credit reasoned, applied evaluation:

(a) Laptop drive — recommend flash/SSD:

No moving parts → durable and shock-resistant, important for journalists filming on location (1).

Fast access, low power (better battery life), silent and lightweight/portable for travel (1).

Trade-off acknowledged: more expensive per GB and finite write cycles, but capacity is sufficient for a laptop (1).

(b) Archiving large rarely-accessed footage — recommend magnetic (HDD or tape):

Very large capacity at low cost per GB, ideal for huge volumes of old footage (1).

Tape in particular is cheap and reliable for long-term archive; slow serial access matters little as the data is rarely retrieved (1).

Trade-off acknowledged: slower with moving parts/fragility, acceptable for infrequent backup access (1).

Evaluation / comparison (higher band):

Explicitly weighs cost vs speed vs durability vs access pattern for each scenario and reaches a clear justified choice for both (1).

Considers and rejects optical with reason (cheap and portable but far too low capacity for large archives) (1).

Level guide: L3 (7–8) sustained, well-applied evaluation with justified choices for both purposes; L2 (4–6) some application and comparison; L1 (1–3) basic descriptions with limited application.

Examiner tip: For both parts, link each storage characteristic directly to a stated requirement (travel/durability; volume/cost/rare access) — generic pros/cons without application stays at the lower bands.