# Worksheet — 1.1 Processors, Input, Output & Storage

OCR A Level Computer Science (H446) — Component 01, Section 1.1 (1.1.1 processor, 1.1.2 processor types, 1.1.3 I/O & storage).

Name: __  Date: __

Time: ~30 min. Write your answers in the spaces; check against the answer key at the end.

## 1. Key-term match

Match each term (1–8) to its definition (A–H). Write the letter on the line.

Terms

Register .......... ____

Cache .......... ____

Pipelining .......... ____

Core .......... ____

Volatile .......... ____

Virtual memory .......... ____

GPU .......... ____

Control Unit .......... ____

Definitions

A. Overlapping the fetch, decode and execute of consecutive instructions to raise throughput.

B. Loses its contents when power is removed.

C. Small, very fast store inside the CPU that holds data during processing.

D. Many-core processor that performs the same operation on large data sets in parallel.

E. Use of secondary storage as extra RAM when physical RAM is full.

F. An independent processing unit able to run its own fetch–decode–execute cycle.

G. Small, fast memory on or near the CPU holding frequently used data/instructions.

H. Decodes instructions, generates control signals and synchronises the CPU with the clock.

## 2. Fill in the blanks — the Fetch–Decode–Execute cycle

Complete the passage using the word bank. Each word is used once.

Word bank: opcode · operand · PC · MDR · CIR · data · control · increment · ALU · address

During fetch, the address of the next instruction is copied from the ____ into the MAR.

A read signal is sent on the __ bus, and the instruction returns along the __ bus into the ____.

The processor then __s the PC so it points to the next instruction, and the fetched instruction is copied from the MDR into the __.

During decode, the Control Unit splits the instruction into the __ (the operation to perform) and the __ (the data or ____ to act on).

During execute, the instruction is carried out — for example a calculation is performed by the ____ with the result stored in the accumulator.

## 3. State the register

For each description, name the register (PC, ACC, MAR, MDR or CIR).

a) Holds the address of the next instruction to be fetched. ........................ __

b) Holds the address of the location currently being read from or written to. ........................ __

c) Holds the data or instruction just fetched from (or about to be written to) memory. ........................ __

d) Holds the current instruction while it is being decoded and executed. ........................ __

e) Stores the immediate result of an ALU calculation. ........................ __

## 4. Put the FDE steps in order

Number these fetch-stage steps 1–4 in the order they happen.

____ Read signal placed on the control bus; instruction returns via the data bus into the MDR.

____ The contents of the MDR are copied into the CIR.

____ The address in the PC is copied into the MAR.

____ The PC is incremented (PC := PC + 1).

## 5. Short answer — performance & architectures

a) State two factors that affect CPU performance and explain the effect of each.

b) "Adding more cores always makes a computer faster." Give the caveat that makes this statement only partly true.

c) Explain the key difference between Von Neumann and Harvard architectures, and give one typical use of each.

d) Give two differences between CISC and RISC processors.

e) Explain why a GPU is well suited to graphics and machine-learning work but poor at branch-heavy sequential tasks.

## 6. Storage device selection table

Complete the empty cells. For each storage type, state how it works, one pro, one con and one example use.

| Type | How it works | One pro | One con | Example use |
| --- | --- | --- | --- | --- |
| Magnetic (e.g. HDD, tape) |  |  |  |  |
| Flash/SSD (e.g. SSD, USB, SD) |  |  |  |  |
| Optical (e.g. CD, DVD, Blu-ray) |  |  |  |  |

## 7. Challenge box

Challenge — virtual memory & thrashing.

A user keeps many large applications open and the computer becomes very slow, with the hard disk constantly active.

(i) Explain what virtual memory is and why the OS is using it here.

____________

____________

(ii) Name the problem causing the slowdown and explain why moving pages between RAM and disk is so slow.

____________

____________

(iii) Suggest one hardware change that would reduce the problem, and justify it.

____________

## Answer key

1. Key-term match

1–C, 2–G, 3–A, 4–F, 5–B, 6–E, 7–D, 8–H.

2. Fill in the blanks (FDE)

…copied from the PC into the MAR. A read signal is sent on the control bus, and the instruction returns along the data bus into the MDR. The processor then increments the PC… copied from the MDR into the CIR.

Decode: splits the instruction into the opcode (the operation) and the operand (the data or address).

Execute: a calculation is performed by the ALU, result stored in the accumulator.

3. State the register

a) PC  b) MAR  c) MDR  d) CIR  e) ACC

4. Put the FDE steps in order

1 — The address in the PC is copied into the MAR.

2 — Read signal placed on the control bus; instruction returns via the data bus into the MDR.

3 — The PC is incremented (PC := PC + 1).

4 — The contents of the MDR are copied into the CIR.

(Steps 3 and 4 may be quoted either way round; the key points are that the PC must be incremented and the MDR must be copied into the CIR.)

5. Short answer — performance & architectures

a) Any two, e.g.:

Clock speed — pulses per second synchronise the CPU; a higher clock speed means more FDE cycles per second (limited by heat/power).

Cores — each core has its own FDE cycle, so multiple instructions can be processed simultaneously.

Cache — small, very fast memory on/near the CPU holding frequent/recent data, reducing slow fetches from RAM.

Pipelining — overlaps fetch/decode/execute of consecutive instructions, increasing throughput.

b) Extra cores only help if the task can be split into parallel parts and the software is written to use multiple cores; inherently sequential tasks gain little.

c) Von Neumann uses one shared memory and bus for both data and instructions (simpler/cheaper but causes a bottleneck because it cannot fetch an instruction and read/write data at the same time); Harvard uses separate memories/buses for data and instructions, allowing simultaneous access. Use: Von Neumann — general-purpose PCs/laptops; Harvard — embedded systems / microcontrollers / DSPs.

d) Any two, e.g.: CISC has many complex, variable-length, multi-cycle instructions with complexity in hardware; RISC has few simple fixed-length instructions aiming for one per cycle with complexity in the compiler. RISC pipelines more easily and uses less power (good for battery devices/ARM); CISC is typical of x86 desktops.

e) A GPU has thousands of small cores that apply the same operation to many data items in parallel (data parallelism), which suits pixels/vertices and large ML matrices. Branch-heavy, dependency-laden or inherently sequential work cannot be parallelised this way (each step needs the previous result), so a CPU handles it better.

6. Storage device selection table (example answers)

| Type | How it works | One pro | One con | Example use |
| --- | --- | --- | --- | --- |
| Magnetic | Magnetised patterns/polarity on a spinning platter or tape, read/written by a R/W head | Huge capacity, low cost per GB | Slow (moving parts), fragile | Bulk/backup storage; tape archive |
| Flash/SSD | EEPROM cells trap electrons; no moving parts | Fast, silent, low power, durable | Costlier per GB, finite write cycles | Laptop main drive; USB stick; SD card |
| Optical | Laser detects pits and lands via reflected light | Very cheap, portable, long shelf life | Low capacity, slow, easily scratched | Distributing films/software; long-term disc archive |

7. Challenge box

(i) Virtual memory uses secondary storage (a swap/paging file) as extra RAM when physical RAM is full; pages are moved out to disk and back as needed. It is being used because the open applications need more memory than the installed RAM provides.

(ii) The problem is disk thrashing — excessive paging. It is slow because secondary storage (disk) is far slower than RAM, so constantly swapping pages in and out wastes most of the CPU's time waiting.

(iii) e.g. Install more RAM so fewer pages need swapping to disk (less/no paging), or fit an SSD so any paging that does occur is much faster than on an HDD.