# Homework — 1.1.2 Types of Processor - ANSWERS

## Mark scheme

Part A (14 marks)

1. [3] — 2 correct cells per mark: (a) large set of complex instructions; (b) fixed length; (c) usually one cycle; (d) in the software/compiler; (e) longer programs, more RAM needed; (f) higher.

2. [3] — 2 correct rows per mark: Multicore processor = D; Parallel processing = B; GPU = C; Co-processor = A; Pipelining = F; RISC = E. (G and H are distractors: Von Neumann architecture and cache.)

3. [3] — 1 mark per statement correctly fixed:

1: A GPU contains a very large number of simpler cores — far more than a typical CPU.

2: Each core carries out the same instruction on a different item of data at the same time.

3: GPUs are increasingly used for general purpose (non-graphics) computing, e.g. machine learning, scientific modelling/simulation, data mining.

4. [2] — 1 mark per valid reason (max 2), linked to a phone: RISC uses less power → longer battery life; simpler instructions run efficiently/cool in a small device without a fan; easier to pipeline → good performance per watt; cheaper/simpler hardware keeps the handset cost down. (Generic "RISC is simpler" with no link caps at 1.)

5. [3] — 1 mark per row (correct verdict; for false statements the correction is required):

| Statement | True / False | Correction |
| --- | --- | --- |
| More cores = automatically faster | False | The software must be written to use multiple cores / the task must be capable of being split (parallelised); otherwise extra cores give little or no benefit |
| Co-processor takes over specific tasks | True | — |
| Parallel processing is strictly sequential | False | In parallel processing several instructions are processed simultaneously (strictly one-after-another is serial processing) |

Part B (6 marks)

6. [3] — 1 mark: Von Neumann uses a single shared memory and bus for both data and instructions. 1 mark: so it cannot fetch an instruction and read/write data at the same time (they compete for the one bus) — the bottleneck. 1 mark: Harvard uses separate memories and buses for data and instructions, so instruction fetch and data access can happen simultaneously.

7. [2] — 1 mark each (factor + effect), max 2: clock speed — more pulses/sec → more FDE cycles/sec; cache size — more frequently used data/instructions held near the CPU → fewer slow RAM fetches; pipelining — overlaps FDE stages → higher throughput. (Accept word/bus width.)

8. [1] — CIR (Current Instruction Register).

Total: 14 + 6 = 20