# Quiz — 1.1.2 Types of Processor

Name: ______________________   Date: __________   Mark: ______ / 25

OCR H446 · Component 01 · 1.1.2

## Section A — Multiple choice (8 marks)

1. Compared with CISC, RISC processors typically have:

A. Many complex, variable-length instructions

B. Few simple, fixed-length instructions aiming for one cycle each

C. All complexity built into the hardware

D. Higher power consumption

Your answer: ☐

2. In a CISC processor, the complexity of executing instructions is mainly handled by the:

A. Compiler / software

B. Hardware

C. Cache

D. Address bus

Your answer: ☐

3. Which type of processor is most commonly found in mobile phones and embedded devices because of its low power use?

A. CISC (x86)

B. RISC (ARM)

C. Von Neumann only

D. GPU only

Your answer: ☐

4. A GPU is best suited to:

A. Highly sequential, branch-heavy tasks

B. Applying the same operation to large blocks of data in parallel

C. Running the operating system kernel

D. Storing data permanently when the power is off

Your answer: ☐

5. Which of the following is an example of GPGPU (general-purpose computing on a GPU)?

A. Rendering pixels for a 3D game

B. Training a neural network / machine learning

C. Decoding a single instruction in the CIR

D. Holding the BIOS firmware

Your answer: ☐

6. A task will not benefit much from parallel processing when:

A. It can be split into many independent sub-tasks

B. Each step depends on the result of the previous step (data dependency)

C. The software is written to use multiple threads

D. There are many cores available

Your answer: ☐

7. Which statement about adding more cores is correct?

A. More cores always doubles performance

B. More cores only improve performance if the task can be parallelised and the software supports it

C. More cores reduce clock speed proportionally

D. More cores remove the need for cache

Your answer: ☐

8. Which feature of RISC processors makes pipelining easier than in CISC processors?

A. Instructions are simple, fixed-length and typically take one cycle each

B. Instructions are complex and variable in length

C. RISC processors have no registers

D. RISC processors store instructions and data in separate memories

Your answer: ☐

## Section B — Short answer (12 marks)

9. Explain two differences between CISC and RISC processors. [2]

10. Explain why a GPU can render 3D graphics so quickly. [2]

11. Define the term multicore processor. [1]

12. A program sorts a list and then searches it. The search cannot start until the sort has finished. Explain why splitting this work across multiple cores gives limited benefit. [2]

13. State three uses of GPUs other than producing graphics. [3]

14. A manufacturer chooses a RISC processor for a battery-powered fitness tracker. Explain why RISC is well suited to this device. [2]

## Section C — Exam-style question (5 marks)

15. A research team needs to run large-scale scientific simulations that apply the same mathematical calculation to millions of data points. They are deciding whether to invest in a powerful multicore CPU or a high-end GPU.

Discuss which processor type is better suited to this workload and justify your answer. (AO2/AO3) [5]