# Knowledge Organiser — 1.1 Processors, Input, Output & Storage

OCR A Level Computer Science (H446) — Component 01, Section 1.1 (1.1.1 processor, 1.1.2 processor types, 1.1.3 I/O & storage).

## Processor components & registers

ALU — arithmetic (+ − × ÷), logic (AND/OR/NOT, compare) and shifts; result → ACC.

Control Unit (CU) — decodes instructions, generates control signals, manages data flow on buses, synchronises with the clock.

| Register | Full name | Role |
| --- | --- | --- |
| PC | Program Counter | Address of the next instruction; incremented each cycle |
| MAR | Memory Address Register | Address of location to read/write |
| MDR | Memory Data Register | Data/instruction just fetched or about to be written |
| CIR | Current Instruction Register | Holds current instruction while decoded/executed |
| ACC | Accumulator | Stores immediate result of ALU calculations |

MAR width → max addressable locations (2ⁿ). MDR width ≈ data bus width (throughput/fetch).

## Buses

| Bus | Direction | Carries |
| --- | --- | --- |
| Address | Unidirectional (CPU→mem) | Memory address (width → 2ⁿ addressable) |
| Data | Bidirectional | Data/instructions |
| Control | Bidirectional | Control signals (read/write, clock, interrupt, bus request) |

## Fetch–Decode–Execute cycle

FETCH

PC → MAR (address copied)

Read signal on control bus; instruction returns via data bus → MDR

PC := PC + 1 (incremented)

MDR → CIR

DECODE — CU splits CIR into opcode (operation) + operand (data/address).

EXECUTE — carry out: ALU op (result → ACC) / load–store memory / jump (new address written to PC). Repeat.

Jump/branch = writes new address into PC during execute → causes pipeline branch hazards.

## Factors affecting CPU performance

| Factor | Effect |
| --- | --- |
| Clock speed (Hz) | Pulses/sec sync the CPU; higher = more FDE cycles/sec. Limited by heat & power |
| Cores | Independent units, each own FDE; multicore runs instructions simultaneously — only if task parallelisable & software supports it |
| Cache | Small very-fast memory on/near CPU holding frequent/recent data → fewer slow RAM fetches. Cache miss = slow fetch |
| Pipelining | Overlaps fetch/decode/execute of consecutive instructions → higher throughput |

Cache levels: L1 fastest/smallest (on core) → L2 larger/slower → L3 largest/slowest (often shared, still > RAM). Check order: L1→L2→L3→RAM.

Pipeline hazards: branch (flush prefetched) & data (needs in-flight result); fixed by branch prediction / out-of-order execution.

## Von Neumann vs Harvard

|  | Von Neumann | Harvard |
| --- | --- | --- |
| Memory | One shared (data + instructions) | Separate for data & instructions |
| Buses | Shared | Separate |
| Access | Can't fetch + read data at once (bottleneck) | Simultaneous instruction fetch + data access |
| Cost | Simpler/cheaper/flexible | Complex/costly |
| Use | General-purpose PCs/laptops | Embedded, microcontrollers, DSPs |

Modern CPUs = hybrid (modified Harvard): unified main memory but split L1 instruction/data cache.

## CISC vs RISC

|  | CISC | RISC |
| --- | --- | --- |
| Instructions | Many, complex | Few, simple |
| Length | Variable, multi-cycle | Fixed, aim 1/cycle |
| Complexity in | Hardware | Software/compiler |
| Pipelining | Harder | Easier |
| Power | Higher | Lower (battery devices) |
| Use | x86 desktops | ARM mobile/embedded |

## GPUs & parallel processing

GPU — thousands of small cores; same operation on large data blocks in parallel (data parallelism).

Graphics: render/shade pixels & vertices of 3D scenes simultaneously.

GPGPU (non-graphics): ML / neural-network training, scientific simulation, image/video processing, crypto mining/hashing.

Poor at sequential, branch-heavy, dependency-laden work (that suits CPU).

Multicore: several cores on one chip, independent simultaneous processing.

Parallel processing: multiple computations at once; split task across cores → faster.

Won't parallelise well when: data dependency (step needs prior result) / coordination overhead / inherently sequential algorithm. Limited by the serial fraction that must run sequentially.

## Input / output / storage device selection

No single "right" device — justify against the scenario: environment, user/accessibility, accuracy, speed, cost, durability.

Input: optical mouse, keyboard, capacitive touchscreen, barcode scanner, microphone, sensor.

Output: monitor (LCD/LED), laser printer (fast/high-volume/sharp), inkjet (cheap colour/photos/low volume), 3D printer (layer-by-layer), speakers, actuator.

Tie each characteristic to a stated requirement (e.g. laser for high-volume office; inkjet for occasional home photos).

## Storage: magnetic / flash / optical

Secondary storage = non-volatile (retains data without power).

| Type | How it works | Pros | Cons |
| --- | --- | --- | --- |
| Magnetic (HDD, tape) | Magnetised patterns/polarity on spinning platter/tape; R/W head | Huge capacity, low £/GB | Slow (moving parts), fragile, power/noise |
| Flash/SSD (SSD, USB, SD) | EEPROM; traps electrons in cells; no moving parts | Fast, silent, low power, durable, portable | Costlier/GB, finite write cycles |
| Optical (CD/DVD/Blu-ray) | Laser detects pits & lands (reflected light) | Very cheap, portable, long shelf life | Low capacity, slow, scratches, drives rare |

Tape = serial access (great cheap archive, bad random). HDD/SSD = direct/random access. Flash uses wear levelling.

## RAM vs ROM

|  | RAM | ROM |
| --- | --- | --- |
| Volatility | Volatile (lost on power off) | Non-volatile |
| R/W | Read and write | Normally read only |
| Contents | Running programs/data | Bootstrap/BIOS firmware |

## Virtual storage

Virtual memory: when RAM full, OS uses secondary storage (swap/paging file) as extra RAM; pages moved out/in as needed.

Pro: run more/larger programs than physical RAM allows. Con: disk ≪ RAM speed → excess paging = "disk thrashing".

Virtual/cloud storage: remote servers over network — access anywhere, scalable, off-site backup; needs internet, ongoing cost, security concerns. Read question: which "virtual" is meant.

## Must-know definitions

| Term | Definition |
| --- | --- |
| Register | Small, very fast store inside the CPU holding data during processing |
| Cache | Small fast memory on/near CPU holding frequently used data/instructions |
| Pipelining | Overlapping fetch/decode/execute of consecutive instructions for throughput |
| Core | Independent processing unit able to run its own FDE cycle |
| Parallel processing | Carrying out multiple computations simultaneously across cores |
| Volatile | Loses contents when power removed (e.g. RAM) |
| Virtual memory | Use of secondary storage as extra RAM when physical RAM is full |
| GPU | Many-core processor for the same operation on large data sets in parallel |

## Top exam reminders

MAR = address, MDR = data — never swap them.

In the fetch trace, increment the PC AND copy MDR → CIR (both often missed).

Address bus = unidirectional; data & control buses = bidirectional.

"More cores = faster" only if task parallelises and software supports it — always add the caveat.

Cache is separate fast memory on/near the CPU, not part of RAM. Flash/SSD is non-volatile (unlike RAM).

For "choose a device/storage" questions, link each characteristic to the scenario and actually compare ("whereas") — generic pros/cons caps marks at AO1.