# Revision — 1.1.2 Types of Processor

OCR H446 — one-page revision summary

## Must know

Von Neumann: a single shared memory and bus for both data and instructions — simpler, cheaper, flexible, but can suffer a bottleneck (cannot fetch an instruction and access data at the same time). Used in general-purpose PCs.

Harvard: separate memories and buses for data and instructions, allowing simultaneous instruction fetch and data access — more complex/costly. Used in embedded systems, microcontrollers and DSPs.

Most modern CPUs are modified (hybrid) Harvard: one unified main memory (Von Neumann style) but separate L1 instruction and data caches (Harvard style) close to the core.

CISC: many complex, variable-length, multi-cycle instructions; complexity is in the hardware; higher power; used in x86 desktops.

RISC: few simple, fixed-length instructions aiming for one cycle each; complexity is in the software/compiler; easier to pipeline; lower power; used in ARM mobile/embedded devices.

A GPU has thousands of small cores that apply the same operation to large blocks of data in parallel (data parallelism) — ideal for rendering pixels/vertices of 3D scenes.

GPGPU = using a GPU for non-graphics work: machine-learning/neural-network training, scientific simulation, image/video processing, crypto hashing.

Multicore = two or more independent cores on one chip. Parallel processing splits a task across cores. Benefit is limited by the serial fraction — data dependency, coordination overhead or an inherently sequential algorithm prevent effective parallelisation.

## Key terms

| Term | Definition |
| --- | --- |
| Von Neumann architecture | Single shared memory and bus for data and instructions |
| Harvard architecture | Separate memories and buses for data and instructions |
| CISC | Complex Instruction Set Computer — many complex instructions, complexity in hardware |
| RISC | Reduced Instruction Set Computer — few simple fixed-length instructions, one/cycle |
| GPU | Many-core processor applying one operation to large data sets in parallel |
| GPGPU | Using a GPU for general-purpose (non-graphics) parallel computation |
| Multicore processor | A chip containing two or more independent cores |
| Parallel processing | Carrying out multiple computations simultaneously across cores |

## Worked example / how to — choosing between a multicore CPU and a GPU

Scenario: apply the same calculation to millions of data points.

Identify the workload type — the same operation repeated over huge data with little branching or dependency = highly data-parallel.

Match to the hardware — a GPU has thousands of small cores, so it processes many data points simultaneously; a multicore CPU has fewer, more powerful cores better suited to sequential, branch-heavy work.

Add the caveat — the software must be written to use the GPU; very branchy or dependency-heavy work would suit the CPU.

Conclude — for this uniform, massively parallel task the GPU maximises throughput, so it is the better choice.

## Exam tips

Give distinct differences: memory/buses, bottleneck vs simultaneous access, cost/complexity, typical uses — don't repeat the same point reworded.

Link processor type to a real use: RISC/ARM → mobile & embedded (low power); CISC/x86 → desktops; GPU → graphics/GPGPU.

Explain GPU speed with two ideas: thousands of cores and the same operation on many data items in parallel.

For parallelism questions, name the reason benefit is limited — data dependency / serial fraction — and note software must support multiple threads.