# Week 02 (07 Sep) - 1.1.1 CPU and FDE — Homework

Homework 1.1.1 (/20). Programming: Workbook 1 s1-2.

# Homework — 1.1.1 Structure and Function of the Processor

Name: ______________________   Date: __________   Mark: ______ / 20

OCR H446 — set after the 1.1.1 lesson. Show your working.

## Part A — This week's topic (~14 marks)

1. Complete the table by filling in the full name and role of each register. [4] (AO1)

| Register | Full name | Role |
| --- | --- | --- |
| PC | (a) | (b) |
| ACC | (c) | (d) |
| MAR | (e) | (f) |
| CIR | (g) | (h) |

2. The events of the fetch and decode stages of the fetch–decode–execute cycle are shown below in a jumbled order. Number them 1–5 to show the correct order. [2] (AO1)

| Event | Order (1–5) |
| --- | --- |
| The instruction is transferred from memory to the MDR along the data bus |  |
| The Control Unit decodes the instruction in the CIR, splitting it into opcode and operand |  |
| The address held in the PC is copied into the MAR |  |
| The contents of the MDR are copied into the CIR |  |
| The PC is incremented by 1 |  |

3. A processor uses the instruction set and memory contents below. The PC starts at 0 and the ACC starts at 0. Complete the trace table, writing the value held in each column after each instruction has finished executing. The trace stops when HALT is fetched. [3] (AO2)

| Instruction | Meaning |
| --- | --- |
| LOAD n | Copy the contents of memory address n into the ACC |
| ADD n | Add the contents of memory address n to the ACC |
| STORE n | Copy the contents of the ACC into memory address n |
| HALT | Stop execution |

| Address | Contents |
| --- | --- |
| 0 | LOAD 4 |
| 1 | ADD 5 |
| 2 | STORE 6 |
| 3 | HALT |
| 4 | 7 |
| 5 | 5 |
| 6 | 0 |

| PC | CIR | ACC | Contents of address 6 |
| --- | --- | --- | --- |

4. Complete the passage about processor performance using words from the word bank. Not all words are used. [3] (AO1)

Several factors affect the performance of a processor. Increasing the (a) __________ speed increases the number of fetch–decode–execute cycles completed each second. (b) __________ is a small amount of very fast memory built into or close to the CPU; it holds frequently used instructions and data so the processor makes fewer slow accesses to (c) __________. With (d) __________, the processor overlaps the stages of the cycle: while one instruction is being (e) __________, the next is being decoded and a third is being fetched. A processor with more than one (f) __________ can carry out several instructions truly in parallel.

Word bank: cache, clock, core, executed, interrupt, pipelining, RAM, register, ROM

5. A processor in an embedded system has a 16-bit address bus and an 8-bit data bus.

(a) Calculate the maximum number of memory locations that can be directly addressed. Show your working. [1] (AO2)

(b) State one effect on the performance of the processor of widening the data bus to 16 bits. [1] (AO1)

## Part B — Retrieval: keep it fresh (~6 marks)

6. Match each bus to its description by writing the correct letter in the Match column. Two descriptions are not needed. [3] (1.1.1)

| Bus | Match |
| --- | --- |
| Address bus |  |
| Data bus |  |
| Control bus |  |

| Letter | Description |
| --- | --- |
| A | Bidirectional; carries data and instructions between the CPU and memory |
| B | Unidirectional; carries the address of the memory location to be accessed, from the CPU to memory |
| C | Bidirectional; carries signals such as read/write, clock and interrupt |
| D | Unidirectional; carries the results of calculations back to the ACC |
| E | Bidirectional; carries the address of the next instruction from memory to the CPU |

7. Explain what cache memory is and one reason it improves CPU performance. [2] (1.1.1)

8. State what is meant by pipelining in a processor. [1] (1.1.1)