# Flashcard Bank — Week 02 (07 Sep) - 1.1.1 CPU and FDE

Everything taught so far (1 subtopic). Cover the answer column, quiz little and often, tick cards you get right three times in a row.

## 1.1.1

| Question | Answer |
| --- | --- |
| What are the two main internal components of the CPU? | The Arithmetic Logic Unit (ALU) and the Control Unit (CU), plus a set of registers. |
| What does the Arithmetic Logic Unit (ALU) do? | Carries out all arithmetic operations (add, subtract, etc.), logical operations (AND, OR, NOT, comparisons) and shifts; the result is normally placed in the Accumulator. |
| What does the Control Unit (CU) do? | Decodes each instruction, generates control signals to direct other components, manages data flow along the buses, and synchronises operations using the system clock. |
| Define a register. | A small, extremely fast block of memory inside the CPU used to temporarily hold data during processing. |
| What is the role of the Program Counter (PC)? | Holds the address of the next instruction to be fetched; it is incremented during each cycle. |
| What is the role of the Memory Address Register (MAR)? | Holds the address of the memory location currently being read from or written to. |
| What is the role of the Memory Data Register (MDR)? | Holds the data or instruction just fetched from, or about to be written to, memory. |
| What is the role of the Current Instruction Register (CIR)? | Holds the current instruction while it is being decoded and executed. |
| What is the role of the Accumulator (ACC)? | Stores the immediate results of calculations carried out by the ALU. |
| What does the address bus carry and in which direction? | Carries the memory address of the location to read/write; unidirectional (CPU to memory). |
| What does the data bus carry and in which direction? | Carries the actual data/instructions transferred between CPU and memory; bidirectional. |
| What does the control bus carry and in which direction? | Carries control signals (e.g. read/write, clock, interrupt request); bidirectional. |
| List the steps of the FETCH phase of the FDE cycle. | 1) Address in PC copied to MAR; 2) instruction fetched along data bus into MDR (read signal on control bus); 3) PC incremented; 4) contents of MDR copied into CIR. |
| What happens in the DECODE phase of the FDE cycle? | The Control Unit decodes the instruction in the CIR, splitting it into the opcode (the operation) and the operand (the data/address it acts on). |
| What happens in the EXECUTE phase of the FDE cycle? | The decoded instruction is carried out, e.g. the ALU performs a calculation (result to ACC), data is loaded/stored, or the PC is changed (a jump); the cycle then repeats. |
| How is a jump/branch instruction carried out in machine code? | A new address is written directly into the PC during the execute phase, so the next fetch comes from a different location (this implements loops and selection). |
| What is clock speed and how is it measured? | The rate at which the clock generates pulses to synchronise CPU operations, measured in hertz (Hz); e.g. 3.5 GHz = 3.5 billion cycles per second. |
| How does a higher clock speed affect performance, and what limits it? | More FDE cycles per second means faster processing; limited by rising heat dissipation and power consumption. |
| What is a core? | An independent processing unit within a CPU capable of running its own fetch-decode-execute cycle. |
| Why does adding more cores not always increase performance? | Only helps if the software is written to use multiple cores and the task can be split; data dependencies, sequential work and coordination overheads limit gains. |
| What is cache and why does it improve performance? | A small amount of very fast memory on or near the CPU that stores frequently/recently used instructions and data, so the CPU avoids slower fetches from main memory (RAM). |
| Compare L1, L2 and L3 cache. | L1 is smallest and fastest, on the core; L2 is larger but slower; L3 is largest and slowest of the three (still faster than RAM) and is often shared between cores. |
| What is pipelining? | Splitting instruction processing into stages so that while one instruction is executed, the next is decoded and a third is fetched simultaneously, increasing throughput. |
| What is a branch hazard in pipelining? | When a taken jump/branch invalidates instructions already fetched ahead of it, so they must be flushed/discarded; mitigated by branch prediction. |
| Describe Von Neumann architecture. | A single shared memory and bus holds both program instructions and data, accessed over a shared bus. |
| Describe Harvard architecture. | Instructions and data are stored in physically separate memories with separate buses, allowing them to be accessed at the same time. |
| What is the Von Neumann bottleneck? | Because data and instructions share one memory and bus, an instruction cannot be fetched while data is read/written at the same time, limiting throughput. |
| Give a typical use of Von Neumann and of Harvard architecture. | Von Neumann: general-purpose computers (PCs, laptops). Harvard: embedded systems, microcontrollers and DSPs. |