# Homework — 1.1.2 Types of Processor — ANSWERS

## Mark scheme

Part A (14 marks)

1. [3] — 2 correct cells per mark: (a) large set of complex instructions; (b) fixed length; (c) usually one cycle; (d) in the software/compiler; (e) longer programs, more RAM needed; (f) higher.

2. [3] — 2 correct rows per mark: Multicore processor = D; Parallel processing = B; GPU = C; Co-processor = A; Pipelining = F; RISC = E. (G and H are distractors: Von Neumann architecture and cache.)

3. [3] — 1 mark per statement correctly fixed:

1: A GPU contains a very large number of simpler cores — far more than a typical CPU.

2: Each core carries out the same instruction on a different item of data at the same time.

3: GPUs are increasingly used for general purpose (non-graphics) computing, e.g. machine learning, scientific modelling/simulation, data mining.

4. [2] — 1 mark per valid reason (max 2), linked to a phone: RISC uses less power → longer battery life; simpler instructions run efficiently/cool in a small device without a fan; easier to pipeline → good performance per watt; cheaper/simpler hardware keeps the handset cost down. (Generic "RISC is simpler" with no link caps at 1.)

5. [3] — 1 mark per row (correct verdict; for false statements the correction is required):

| Statement | True / False | Correction |
| --- | --- | --- |
| More cores = automatically faster | False | The software must be written to use multiple cores / the task must be capable of being split (parallelised); otherwise extra cores give little or no benefit |
| Co-processor takes over specific tasks | True | — |
| Parallel processing is strictly sequential | False | In parallel processing several instructions are processed simultaneously (strictly one-after-another is serial processing) |

Part B (6 marks)

6. [3] — 1 mark: Von Neumann uses a single shared memory and bus for both data and instructions. 1 mark: so it cannot fetch an instruction and read/write data at the same time (they compete for the one bus) — the bottleneck. 1 mark: Harvard uses separate memories and buses for data and instructions, so instruction fetch and data access can happen simultaneously.

7. [2] — 1 mark each (factor + effect), max 2: clock speed — more pulses/sec → more FDE cycles/sec; cache size — more frequently used data/instructions held near the CPU → fewer slow RAM fetches; pipelining — overlaps FDE stages → higher throughput. (Accept word/bus width.)

8. [1] — CIR (Current Instruction Register).

Total: 14 + 6 = 20

# Quiz — 1.1.1 Structure and Function of the Processor — ANSWERS

## Answer key

Section A (8 marks — 1 each)

C — The Program Counter always holds the address of the next instruction; it is incremented each cycle.

B — The Memory Data Register holds data/instruction just fetched or about to be written. (Remember: MAR = address, MDR = data.)

B — The address bus is unidirectional, CPU → memory only.

C — After PC → MAR, a read signal is placed on the control bus and the instruction returns via the data bus into the MDR.

C — The CU decodes instructions and generates control signals to synchronise and direct the CPU.

B — Cache is small, very fast memory on/near the CPU (not part of RAM) holding frequently/recently used data.

B — More cores help only if the task is parallelisable and software supports it.

B — Pipelining overlaps fetch/decode/execute of consecutive instructions to raise throughput.

Section B (12 marks)

[3] — 1 mark each (full name + role):

MAR = Memory Address Register — holds the address of the location to be read from / written to.

MDR = Memory Data Register — holds the data/instruction just fetched, or about to be written, to memory.

CIR = Current Instruction Register — holds the current instruction while it is decoded/executed.

[2] — 1 mark each: Performs arithmetic operations (+ − × ÷); performs logic operations / comparisons (AND, OR, NOT, compare) and shifts; (result is placed in the ACC). Any two distinct points.

[2] — 2¹⁶ (1 mark for method 2ⁿ) = 65,536 locations (1 mark for correct answer).

[2] — 1 mark: higher clock speed = more pulses/cycles per second, so more fetch–decode–execute cycles completed per second / more instructions processed per second. 1 mark: limited by heat / power consumption (overheating).

[2] — 1 mark: L1 is smaller and faster (on the core); L3 is larger but slower (often shared) — though still faster than RAM. 1 mark: checked in order L1 → L2 → L3 → RAM.

[1] — A new address is written into the PC (so execution continues elsewhere); the instructions already prefetched into the pipeline must be flushed/discarded (branch hazard), wasting cycles.

Section C (5 marks)

[5] — Level-based / point marking, award up to 5 for a balanced, applied discussion. Indicative content:

Higher clock speed (X) means each individual core completes more FDE cycles per second — good for the sequential / single-threaded parts of a game (e.g. game logic, physics dependencies).

More cores (Y) allow parallel processing — multiple tasks (rendering prep, AI, audio, physics) run simultaneously — but only if the game engine is written to use multiple threads.

Larger cache (Y) reduces slow RAM fetches (fewer cache misses), improving throughput for frequently accessed data/instructions.

Trade-off: if the game is poorly multi-threaded, X's higher clock speed may win; modern game engines are increasingly parallel, favouring Y.

Justified conclusion linking the choice to the workload (e.g. "Y is likely better for a modern, multi-threaded engine because…").

Max 4 if no justified judgement/conclusion is made.

Total: 8 + 12 + 5 = 25